Custom Subnet Masks

Problem 1

Number of needed subnets 14

Number of needed usable hosts 14

Network Address 192.10.10.0

Address class 	C

Default subnet mask 	255 . 255 . 255 . 0

Custom subnet mask 	255 . 255 . 255 . 240

Total number of subnets 			16			 Total number of host addresses 			16			 Number of usable addresses 		14		 Number of bits borrowed 	4

# Show your work for Problem 1 in the space below.

Number of Subnets	-

## 192 . 10 . 10 .	0	0	0	0

## 0	0	0	0

Number of

Add the binary value numbers to the left of the line to create the custom subnet mask.

128

| 256 128 64 | 32 | 16 | 8 | 4 | 2 | - Hosts |
| --- | --- | --- | --- | --- | --- | --- |
| 2 4 8 | 16 | 32 | 64 128 256 |  |  |  |
| 128 64 32 | 16 | 8 | 4 | 2 | 1 | - Binary values |

64

32

+16 240

16

-2 14

Observe the total number of hosts.

Subtract 2 for the number of usable hosts.

Custom Subnet Masks

Problem 2

Number of needed subnets 1000

Number of needed usable hosts 60

Network Address 165.100.0.0

Address class 	B

Default subnet mask 	255 . 255 . 0 . 0

Custom subnet mask 	255 . 255 . 255 . 192

Total number of subnets 		1,024			 Total number of host addresses 			64			 Number of usable addresses 		 62		 Number of bits borrowed 	10

512

1,024

2,048

4,096

8,192

16,384

32,768

Show your work for Problem 2 in the space below.

65,536

Number of

1024

512

Hosts -

. 256 128

65,536

32,768

16,384

8,192

4,096

2048

64 32  16	8	4	2

Add the binary value numbers to the left of the line to create the custom subnet mask.

128

| Number of Subnets | - | 2 | 4 8 | 16 32 | 64 | 128 256. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary values | - | 128 64 32 |  | 16 8 | 4 | 2 1 . 128 | 64 | 32 16 | 8 | 4 | 2 | 1 |
| 165 . 100 | . | 0 0 0 |  | 0 0 | 0 | 0 0 . 0 | 0 | 0 0 | 0 | 0 | 0 | 0 |

64

32

16

8

4

2

128

+64

192

64

-2 62

Observe the total number of hosts.

Subtract 2 for the number of usable hosts.

+1 255

Problem 3

Custom Subnet Masks

/26 indicates the total number of bits used for the network and subnetwork portion of the address. All bits remaining belong to the host portion of the address.

Network Address 148.75.0.0 /26

Address class 	B

Default subnet mask 	255 . 255 . 0 . 0

Custom subnet mask 	255 . 255 . 255 . 192

Total number of subnets 		1,024			 Total number of host addresses 			64			 32 bits - /26 = 6 2^6 = 64

Number of usable addresses 		 62		 Number of bits borrowed 	10			 class = 16 /26 = 10 = borrowed bits

512

1,024

2,048

4,096

8,192

16,384

32,768

65,536

Show your work for Problem 3 in the space below.

Number of

1024

512

Hosts -

. 256 128

65,536

32,768

16,384

8,192

4,096

2048

64 32  16	8	4	2

Add the binary value numbers to the left of the line to create the custom subnet mask.

128

| Number of Subnets | - | 2 | 4 8 | 16 32 | 64 | 128 256. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary values | - | 128 64 32 |  | 16 8 | 4 | 2 1 . 128 | 64 | 32 16 | 8 | 4 | 2 | 1 |
| 148 . 75 | . | 0 0 0 |  | 0 0 | 0 | 0 0 . 0 | 0 | 0 0 | 0 | 0 | 0 | 0 |

64

32

16

8

4

128

+64

192

64

-2

62

Observe the total number of hosts.

Subtract 2 for the number of usable hosts.

2

+1 255

1024

-2 1,022

Subtract 2 for the total number of subnets to get the usable number of subnets.

Custom Subnet Masks

Problem 7

Number of needed subnets 2000

Number of needed usable hosts 15

Network Address 178.100.0.0

Address class Class B

Default subnet mask 255.255.0.0

Custom subnet mask 255.255.255.224

Total number of subnets 2048

Total number of host addresses 32

Number of usable addresses 30

Number of bits borrowed 11

# Show your work for Problem 7 in the space below.

65,536

Number of

Hosts -

32,768

16,384

8,192

4,096

2048

1024

512

512

1,024

2,048

4,096

8,192

16,384

32,768

Number of

. 256 128 64| 32 16	8	4	2

Subnets	-	2	4	8	16 32 64	128 256.

65,536

Binary values -

128 64  32	16	8	4	2	1 .

128 64 32 | 16	8	4	2	1

## 178 . 100 . 0	0	0	0	0	0	0	0 .	0	0	0 |	0	0	0	0	0

Address Class: 128 to 191 = Class B

Default Subnet Mask: Class B default  is 255.255.0.0

Custom Subnet Mask: 2048 so the line cuts off at  32 binary so 128 + 64 + 32 = 224 = 255.255.255.224

Total Number of Subnets: 2^11 = 2048

Total Number of Host Addresses : Class B  - 16 bits for hosts - 2 ^5 = 32

Number of Usable Addresses : 32 – 2 = 30

Number of Bits : 11 because we get it from  2^ 11 OR count number of 0s before the line

Custom Subnet Masks

Problem 15

Number of needed usable hosts 50

Network Address 172.59.0.0

Address class: Class B

Default subnet mask 255.255.0.0

Custom subnet mask 255.255.255.192

Total number of subnets 1024

Total number of host addresses 64

Number of usable addresses 62

Number of bits borrowed 10

# Show your work for Problem 15 in the space below.

Subnets -                    2     4     8   16   32 64 | 128 256

------------------------------------------

Binary Values -        128   64   32   16  8   4  |  2   1

-----------------------------------------

175 . 59 .                   0      0      0     0   0   0 |   0   0  . 0 0 0 0 0 0 0 0

| H | 65536 | 32768 | 16384 | 8192 | 4096 | 2014 | 1024 | 512 | .256 | 128 | ¦64 | 32 | 16 | 8 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | .512 | 1024 | ¦2048 | 4096 | 8192 | 16384 | 32768 | 65356 |
| B | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | .128 | 64 | ¦32 | 16 | 8 | 4 | 2 | 1 |
| IP | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | . 0 | 0 | ¦0 | 0 | 0 | 0 | 0 | 0 |

Address Class: 128 to 191 = Class B

Default Subnet Mask: Class B default  is 255.255.0.0

Custom subnet mask : 128 + 64 = 255.255.255.192

Total number of subnets: 1024, last subnet before the line

Total number of host addresses: 2 ^ 64

Number of usable addresses: 64 – 2 = 62

Number of bits: 10 because there’s 10 0s before the line

Subnetting

Problem 1

Number of needed subnets 14

Number of needed usable hosts 14

Network Address 192.10.10.0

Address class 	C

Default subnet mask 	255 . 255 . 255 . 0

Custom subnet mask 	255 . 255 . 255 . 240

Total number of subnets 			16			 Total number of host addresses 			16			 Number of usable addresses 		14		 Number of bits borrowed 	4

What is the 4th

subnet range?  192.10.10.48	to	192.10.10.63

What is the subnet number

for the 8th subnet?

What is the subnet broadcast address for the 13th subnet?

What are the assignable addresses for the 9th

192 . 10 . 10 . 112

192 . 10 . 10 . 207

subnet?  192.10.10.129	to	192.10.10.142

# Show your work for Problem 1 in the space below.

Number of

Number of

256 128 64 32	16	8	4	2	-	Hosts

Subnets	-	2	4	8  16	32	64 128 256

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 - | Binary values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 192. 10 . 10 . 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |

(0) 0

(1) 0

(2) 0

(3) 0

(4) 0

(5) 0

(6) 0

(7) 0

(8)  1

(9)  1

(10)  1

(11) 1

(12)  1

(13)  1

(14)  1

(15)  1

0	0 0

0	0	1

0	1	0

0	1	1

1	0  0

1	0	1

1	1	0

1	1	1

0	0 0

0	0	1

0	1	0

0	1	1

1	0  0

1	0	1

1	1	0

1	1	1

192.10.10.0	to	192.10.10.15

192.10.10.16	to	192.10.10.31

192.10.10.32	to	192.10.10.47

192.10.10.48	to	192.10.10.63

192.10.10.64	to	192.10.10.79

192.10.10.80	to	192.10.10.95

192.10.10.96	to	192.10.10.111

192.10.10.112	to	192.10.10.127

192.10.10.128	to	192.10.10.143

192.10.10.144	to	192.10.10.159

192.10.10.160	to	192.10.10.175

192.10.10.176	to	192.10.10.191

192.10.10.192	to	192.10.10.207

192.10.10.208	to	192.10.10.223

192.10.10.224	to	192.10.10.239

192.10.10.240	to	192.10.10.255

Custom subnet

mask

128

64

32

+16 240

Usable subnets

16

-2 14

16

Usable hosts -2

14

The binary value of the last bit borrowed is the range. In this problem the range is 16.

The first address in each subnet range is the subnet number.

The last address in each subnet range is the subnet broadcast address.

Subnetting

Problem 2

Number of needed subnets 1000

Number of needed usable hosts 60

Network Address 165.100.0.0

Address class 	B

Default subnet mask 	255 . 255 . 0 . 0

Custom subnet mask 	255 . 255 . 255 . 192

Total number of subnets 		1,024			 Total number of host addresses 			64			 Number of usable addresses 		 62		 Number of bits borrowed 	10

What is the 15th

subnet range?  165.100.3.128	to	165.100.3.191

What is the subnet number

for the 6th subnet?

What is the subnet broadcast address for

the 6th subnet?

What are the assignable addresses for the 9th

165 . 100 . 1 . 64

165 . 100 . 1 . 127

subnet?  165.100.2.1	to	165.100.0.62

65,536

Number of

Hosts -

32,768

16,384

8,192

4,096

2048

1024

512

512

1,024

2,048

4,096

8,192

16,384

32,768

Number of

. 256 128

65,536

64 32  16	8	4	2

Subnets	-	2	4	8	16 32 64	128 256.

Binary values - 128 64 32	16	8	4	2	1 .  128 64 32 16	8	4	2	1

## 165 . 100 . 0	0	0	0	0	0	0	0 .	0	0	0	0	0	0	0	0

64

Usable -2

hosts 62

128

64

32

16

(0)

(1)

(2)

(3)

(4)

(5)

.	0

1

1

1

0

1

1 .	0	0

1 .	0	1

165.100.0.0	to	165.100.0.63

165.100.0.64	to	165.100.0.127

165.100.0.128	to	165.100.0.191

165.100.0.192	to	165.100.0.255

165.100.1.0	to	165.100.1.63

165.100.1.64	to	165.100.1.127

Custom

128

8	(6)

1 .	1

0	165.100.1.128	to	165.100.1.191

subnet mask  +64

192

4	(7)

2	(8)

1 .	1	1

1	0 .	0	0

165.100.1.192	to	165.100.1.255

165.100.2.0	to	165.100.0.63

The binary value of the last bit borrowed is the range. In this problem the range is 64.

+1

255

(9)  1	0 .	0	1

(10)  1	0 .	1	0

165.100.2.64	to	165.100.0.127

165.100.2.128	to	165.100.0.191

The first address in each subnet range is the subnet number.

The last address in each subnet range is the subnet broadcast address.

(11) 1	0 .	1	1

1

1 .

0

0

(12)

(13)  1	1 .	0	1

(14)  1	1 .	1	0

(15)  1	1 .	1	1

165.100.2.192	to	165.100.0.255

165.100.3.0	to	165.100.3.63

165.100.3.64	to	165.100.3.127

165.100.3.128	to	165.100.3.191

165.100.3.192	to	165.100.3.255

Down to

Subnetting

Problem 11

Number of needed usable hosts 8,000

Network Address 135.70.0.0

Address class Class B

Default subnet mask 255.255.0.0

Custom subnet mask 255.255.255.224

Total number of subnets 8

Total number of host addresses 8192

Number of usable addresses 8190

Number of bits borrowed 3

What is the 6th subnet range? 135.70.0.160 to 135.70.0.191

What is the subnet number for the 7th subnet? 135.70.0.192

What is the subnet broadcast address for the 3rd subnet? 135.70.0.95

What are the assignable addresses for the 5th subnet? 135.70.0.129 to 135.70.0.158

# Show your work for Problem 11 in the space below.

| H | 65536 | 32768 | 16384 | ¦8192 | 4096 | 2014 | 1024 | 512 | .256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | 2 | 4 | 8 | ¦16 | 32 | 64 | 128 | 256 | .512 | 1024 | 2048 | 4096 | 8192 | 16384 | 32768 | 65356 |
| B | 128 | 64 | 32 | ¦16 | 8 | 4 | 2 | 1 | .128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| IP | 0 | 0 | 0 | ¦0 | 0 | 0 | 0 | 0 | . 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| (0) | 0 | 0 | 0 ¦ | 135.70.0.0 | To | 135.70.0.31 |
| --- | --- | --- | --- | --- | --- | --- |
| (1) | 0 | 0 | 1 ¦ | 135.70.0.32 | To | 135.70.0.63 |
| (2) | 0 | 1 | 0 ¦ | 135.70.0.64 | To | 135.70.0.95 |
| (3) | 0 | 1 | 1 ¦ | 135.70.0.96 | To | 135.70.0.127 |
| (4) | 1 | 0 | 0 ¦ | 135.70.0.128 | To | 135.70.0.159 |
| (5) | 1 | 0 | 1 ¦ | 135.70.0.160 | To | 135.70.0.191 |
| (6) | 1 | 1 | 0 ¦ | 135.70.0.192 | To | 135.70.0.223 |
| (7) | 1 | 1 | 1 ¦ | 135.70.0.224 | To | 135.70.0.255 |

Address Class: 128 to 191 = Class B

Default Subnet Mask: Class B default  is 255.255.0.0

Custom subnet mask : 128 + 64 + 32 = 224 = 255.255.255.224

Total number of subnets: 8 is left of line

Total number of host addresses: 2^13 = 8192

Number of usable addresses: 8192 – 2 = 8190

Number of bits: 3 0’s left of line

How to do it if its /26

/26

1111 1111 1111 1111 1111 1111 1100 0000

2^7 + 2^6 = 192

255.255.255.192

Subnetting

Problem 12

Number of needed usable hosts 45

Network Address 198.125.50.0

Address class Class C

Default subnet mask 255.255.255.0

Custom subnet mask 255.255.255.192

Total number of subnets 4

Total number of host addresses 64

Number of usable addresses 62

Number of bits borrowed 2

What is the 2nd

subnet range? 198.125.50.64 to 198.125.50.127

What is the subnet number

for the 2nd subnet? 198.125.50.64

What is the subnet broadcast address for

the 4th subnet? 198.125.50.255

What are the assignable addresses for the 3rd

subnet? 198.125.50.129 to 198.125.50.190

# Show your work for Problem 12 in the space below.

| H | 256 | 128 | ¦ 64 | 32 | 16 | 8 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | 2 | 4 | ¦ 8 | 16 | 32 | 64 | 128 | 256 |
| B | 128 | 64 | ¦ 32 | 16 | 8 | 4 | 2 | 1 |
| IP | 0 | 0 | ¦ 0 | 0 | 0 | 0 | 0 | 0 |

| (0) | 0 | 0 ¦ | 198.125.50.0 | To | 198.125.50.63 |
| --- | --- | --- | --- | --- | --- |
| (1) | 0 | 1 ¦ | 198.125.50.64 | To | 198.125.50.127 |
| (2) | 1 | 0 ¦ | 198.125.50.128 | To | 198.125.50.191 |
| (3) | 1 | 1 ¦ | 198.125.50.192 | To | 198.125.50.255 |

Address Class: 192 to 223 = Class C

Default Subnet Mask: Class C default  is 255.255.0.0

Custom subnet mask : 128 + 64 = 192 = 255.255.255.192

Total number of subnets: 4 is left of line

Total number of host addresses: 2^6 = 64

Number of usable addresses: 64 – 2 = 62

Number of bits: 2 0’s left of line

Practical Subnetting 1

Based on the information in the graphic shown, design a network addressing scheme that will supply the minimum number of subnets, and allow enough extra subnets and hosts for 100% growth in both areas. Circle each subnet on the graphic and answer the questions below.

F0/0

Router A

S0/0/0

S0/0/1

F0/0

F0/1

Router B

Marketing 24 Hosts

Reasearch 60 Hosts

Management 15 Hosts

IP Address 172.16.0.0

Address class Custom subnet mask

Minimum number of subnets needed

B

255.255.224.0

4

Extra subnets required for 100% growth	+	4

(Round up to the next whole number)

Total number of subnets needed	=	8

Number of host addresses in the largest subnet group

Number of addresses needed for 100% growth in the largest subnet

(Round up to the next whole number)

Total number of address needed for the largest subnet

60

+ 60

= 120

Start with the first subnet and arrange your sub-networks from the largest group to the smallest.

IP address range for Research IP address range for Marketing

IP address range for Management

IP address range for Router A to Router B serial connection

58

172.16.0.0 to 172.31.255

172.16.32.0 to 172.63.255

172.16.64.0 to 172.95.255

172.16.96.0 to 172.127.255

16,384

32,768

65,536

Number of

8,192

2048

1024

512

Hosts -

32,768

16,384

4,096

. 256 128 64 32 16	8	4	2

| Number of Subnets | - |  | 2 | 4 8 | 16 32 | 64 128 256. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary values | - | 128 64 32 |  |  | 16 8 | 4 2 | 1 . 128 64 32 16 | 8 | 4 | 2 | 1 |
| 172 . 16 | . | 0 0 0 |  |  | 0 0 | 0 0 | 0 . 0 0 0 0 | 0 | 0 | 0 | 0 |

4 x1.0

4

60 x1.0

60

(0)	0

(1)	1

(2)	1	0

(3)	1	1

(4) 1	0	0

(5) 1	0	1

(6) 1	1	0

(7) 1	1	1

172.16.0.0

172.16.32.0

172.16.64.0

172.16.96.0

172.16.128.0

172.16.160.0

172.16.192.0

172.16.224.0

to	172.16.31.255

65,536

to	172.16.63.255

to	172.16.95.255

to	172.16.127.255

to	172.16.159.255

to	172.16.191.255

to	172.16.223.255

to	172.16.255.255

512

1,024

2,048

4,096

8,192

Show

your

work

for

Practical

Subnetting

1

in

the

space

below.

59

Practical Subnetting 2

Based on the information in the graphic shown, design a network addressing scheme that will supply the minimum number of hosts per subnet, and allow enough extra subnets and hosts for 30% growth in all areas. Circle each subnet on the graphic and answer the questions below.

Address class Custom subnet mask

Minimum number of subnets needed

Extra subnets required for 30% growth

(Round up to the next whole number)

Total number of subnets needed

Number of host addresses in the largest subnet group

Number of addresses needed for 30% growth in the largest subnet

(Round up to the next whole number)

Total number of address needed for the largest subnet

B

255.255.255.224

5

+	2

=	7

20

+	6

=	26

Start with the first subnet and arrange your sub-networks from the largest group to the smallest.

IP address range for Tech Ed IP address range for English IP address range for Science

IP address range for Router A to Router B serial connection

IP address range for Router A

135.126.0.0 to 135.126.0.31  135.126.0.32 to 135.126.0.63

135.126.0.64 to 135.126.0.95

135.126.0.96 to 135.126.0.127

IP Address 135.126.0.0

S0/0/0

F0/0

Router A	S0/0/1

S0/0/1

F0/0

Router B

F0/1

S0/0/0

Tech Ed Lab 20 Hosts

F0/1

Router C

Science Lab 10 Hosts

English Department 15 Hosts

to Router B serial connection135.126.0.128 to 135.126.0.159

60

65,536

Number of

Hosts -

32,768

16,384

8,192

4,096

2048

1024

512

512

1,024

2,048

4,096

8,192

16,384

32,768

Number of

. 256 128 64 32 16	8	4	2

Subnets	-	2	4	8	16 32 64	128 256.

65,536

Binary values -

128 64  32	16	8	4	2	1 .

128 64 32

16	8	4	2	1

## 135. 126 . 0	0	0	0	0	0	0	0 .	0	0	0	0	0	0	0	0

5

x.3

1.5

(Round up to 2)

20

x.3

6

(0)	.	0

(1)	1

(2)	1	0

(3)	1	1

(4)	1	0	0

(5)	1	0	1

(6)	1	1	0

(7)	1	1	1

(8) 1 .	0	0	0

(9) 1 .	0	0	1

(10) 1 .	0	1	0

(11) 1 .	0	1	1

.

(12) 1	1	0	0

(13) 1 .	1	0	1

(14) 1 .	1	1	0

(15) 1  .	1	1	1

135.126.0.0	to

135.126.0.32	to

135.126.0.64	to

135.126.0.96	to

135.126.0.128	to

135.126.0.160	to

135.126.0.192	to

135.126.0.224 to

135.126.1.0	to

135.126.1.32	to

135.126.1.64	to

135.126.1.96	to

135.126.1.128	to

135.126.1.160	to

135.126.1.192	to

135.1261.224	to

135.126.0.31

135.126.0.63

135.126.0.95

135.126.0.127

135.126.0.159

135.126.0.191

135.126.0.223

135.126.0.255

135.126.1.31

135.126.1.63

135.126.1.95

135.126.1.127

135.126.1.159

135.126.1.191

135.126.1.223

135.126.1.255

Show

your

work

for

Problem 2

in

the

space

below.

61

Practical Subnetting 4

Based on the information in the graphic shown, design a network addressing scheme that will supply the minimum number of subnets, and allow enough extra subnets and hosts for 70% growth in all areas. Circle each subnet on the graphic and answer the questions below.

IP Address 135.126.0.0

F0/0	S0/0/0

Router A

S0/0/1

S0/0/1

F0/0

S0/0/0

Router B

Dallas

F0/1

Router C	F0/0

150 Hosts	New York

Washington D.C. 220 Hosts

325 Hosts

Address class = Class B

Custom subnet mask = 255.255.240.0

Minimum number of subnets needed = 5

Extra subnets required for 70% growth = +4

Total number of subnets needed = 9

Number of host addresses in the largest subnet group = 325

Number of addresses needed for

70% growth in the largest subnet = +228

Total number of address

needed for the largest subnet = 553

IP address range for New York = 135.126.0.0 to 135.126.15.255

IP address range for Washington D. C. = 135.126.16.0 to 135.126.31.255

IP address range for Dallas = 135.126.32.0 to 135.126.47.255

IP address range for Router A

to Router B serial connection = 135.126.48.0 to 135.126.63.255

IP address range for Router A

to Router C serial connection =135.126.64.0 to 135.126.79.255

# Show your work for Problem 4 in the space below.

| H | 65536 | 32768 | 16384 | 8192 | ¦4096 | 2014 | 1024 | 512 | .256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | 2 | 4 | 8 | 16 | ¦ 32 | 64 | 128 | 256 | .512 | 1024 | 2048 | 4096 | 8192 | 16384 | 32768 | 65356 |
| B | 128 | 64 | 32 | 16 | ¦ 8 | 4 | 2 | 1 | .128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| IP | 0 | 0 | 0 | 0 | ¦ 0 | 0 | 0 | 0 | . 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

0 0 0 | 135.126.0.0 to 135.126.15.255

0 0 1 | 135.126.16.0 to 135.126.31.255

0 1 0 | 135.126.32.0 to 135.126.47.255

0 1 1 | 135.126.48.0 to 135.126.63.255

1 0 0 | 135.126.64.0 to 135.126.79.255

1 0 1 | 135.126.80.0 to 135.126.95.255

1 0 1 | 135.126.96.0 to 135.126.111.255

1 1 1 | 135.126.112.0 to 135.126.127.255

1 0 0 0 | 135.126.128.0 to 135.126.143.255

1 0 0 1 | 135.126.144.0 to 135.126.159.255

1 0 1 0 | 135.126.160.0 to 135.126.175.255

1 0 1 1 | 135.126.176.0 to 135.126.191.255

1 1 0 0 | 135.126.192.0 to 135.126.207.255

1 1 0 1 | 135.126.208.0 to 135.126.223.255

1 1 1 0 | 135.126.224.0 to 135.126.239.255

1 1 1 1 | 135.126.240.0 to 135.126.1255.255

(I done the other workings on a sheet, I am very tired of typing so hopefully this is good enough)

Practical Subnetting 6

Based on the information in the graphic shown, design a network addressing scheme that will supply the minimum number of subnets, and allow enough extra subnets and hosts for 20% growth in all areas. Circle each subnet on the graphic and answer the questions below.

S0/0/0

Router A

F0/0

S0/0/1

S0/0/1

S0/0/0

Technology Building 320 Hosts

S0/0/0

Art & Drama

S0/0/1

Router B

F0/1

75 Hosts

Router C

F0/0

F0/1

Administration 35 Hosts

Science Building 225 Hosts

IP Address 10.0.0.0

Address class = Class A

Custom subnet mask = 255.240.0.0

Minimum number of subnets needed = 7

Extra subnets required for 20% growth = + 2

(Round up to the next whole number)

Total number of subnets needed = 9

IP address range for Technology = 10.0.0.0 to 10.15.255.255

IP address range for Science = 10.16.0.0 to 10.31.255.255

IP address range for Arts & Drama = 10.32.0.0 to 10.47.255.255

IP Address range Administration =10.48.0.0 to 10.63.255.255

IP address range for Router A = 10.64.0.0 to 10.79.255.255

IP address range for Router A to Router C serial connection = 10.80.0.0 to 10.95.255.255

IP address range for Router B to Router C serial connection = 10.96.0.0 to 10.111.255.255

# Show your work for Problem 6 in the space below.

| H | <<<<< | <<<<< | 4,194,304 | 2,097.152 | 1,048,576 | ¦ 524,288 | 262,144 | 131,072 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | 2 | 4 | 8 | 16 | 32 | ¦ 64 | 128 | 256 |
| B | 128 | 64 | 32 | 16 | 8 | ¦ 4 | 2 | 1 |
| IP | 0 | 0 | 0 | 0 | 0 | ¦ 0 | 0 | 0 |

0000 | 10.0.0.0 to 10.15.255.255

0001 | 10.16.0.0 to 10.32.255.255

0010 | 10.32.0.0 to 10.47.255.255

0011 | 10.48.0.0 to 10.63.255.255

0100 | 10.64.0.0 to 10.79.255.255

0101 | 10.80.0.0 to 10.95.255.255

0110 | 10.96.0.0 to 10.111.255.255

0111 | 10.112.0.0 to 10.127.255.255

1000 | 10.128.0.0 to 10.143.255.255

1001 | 10.144.0.0 to 10.159.255.255

1010 | 10.160.0.0 to 10.175.255.255

1011 | 10.176.0.0 to 10.191.255.255

1100 | 10.192.0.0 to 10.207.255.255

1101 | 10.208.0.0 to 10.223.255.255

1110 | 10.224.0.0 to 10.239.255.255

1111 | 10.240.0.0 to 10.255.255.255

(I done the other workings on a sheet, I am very tired of typing so hopefully this is good enough)

(the same thing applies here, typing this stuff is very awkward so I only typed that out…too much workings out, only for the first lab)

69